## SIMATS ENGINEERING

### ASSESSMENT TOOL 3

#### COURSE NAME: Computer Vision with OpenCV	COURSE CODE: DSA02 COURSE OUTCOME COVERED

CO2: Apply image processing and feature extraction techniques, including edge detection, motion estimation, and optical flow, for analyzing images.. (BL3)

Assessment Tool Weightage: 20%

#### QUESTIONS: 2	Total Marks:20

Annexure A

# Class Test

Code of Conduct:

I P. Yaswanth(192524167) (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

P. Yaswanth

#### RUBRICS FOR CALCULATION

| Criteria | Weighta ge | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning | Marks |
| --- | --- | --- | --- | --- | --- | --- |
| Understand ing of Concepts | 25% | Demonstrates thorough, accurate understanding of concepts | Good understanding with minor gaps | Basic understanding with some misconceptions | Poor understanding; major misconceptions |  |
| Explanatio n | 25% | Detailed, well-explained answers with relevant examples | Clear explanation with adequate details | Limited explanation; lacks depth | Poor or unclear explanation |  |
| Applicatio n | 20% | Effectively applies concepts with strong analytical ability | Applies concepts with minor errors | Limited application with weak analysis | No application or incorrect analysis |  |
| Organizati on | 15% | Well-structured answers with logical flow and coherence | Organized with minor issues in flow | Basic structure, lacks clarity | Poor organization, incoherent answers |  |
| Accuracy | 10% | Completely accurate and covers all required points | Mostly accurate with minor omissions | Partially correct with missing points | Incorrect answers with major gaps |  |
| Clarity | 5% | Clear, neat, professional presentation | Understandable with minor issues | Limited clarity, readability issues | Poorly written, difficult to understand |  |

Question 1: Image Preprocessing and Feature Extraction

a) Preprocessing Pipeline (4 Marks)

To accurately identify pest-affected regions in aerial farmland images, image preprocessing is an essential first step. The raw images captured by drones often contain noise, uneven illumination, shadows, and low contrast due to changing environmental conditions. A suitable preprocessing pipeline consists of the following steps:

Step 1: Noise Reduction

The first step is to remove unwanted noise introduced by the drone camera sensor or wind-induced vibrations.

- Method: Gaussian Filter or Bilateral Filter.

- Purpose:

  - Removes random Gaussian noise from the image.

  - Smooths the image while preserving important edges.

  - Bilateral filtering is preferred because it reduces noise without blurring crop boundaries.

Step 2: Illumination Correction

Outdoor images often suffer from uneven lighting because of sunlight intensity, clouds, and shadows.

- Method: Homomorphic Filtering or Adaptive Histogram Equalization (CLAHE).

- Purpose:

  - Reduces the effects of uneven illumination.

  - Makes image brightness more uniform.

  - Improves the visibility of pest-affected regions hidden under shadows.

Step 3: Contrast Enhancement

Healthy and damaged crops may have only slight differences in appearance.

- Method: Histogram Equalization or CLAHE.

- Purpose:

  - Enhances image contrast.

  - Makes texture and color differences more visible.

  - Improves the separation between healthy and infected crop areas.

Step 4: Image Normalization

Images captured at different times may have different brightness levels.

- Method: Min-Max Normalization or Z-score Normalization.

- Purpose:

  - Scales pixel intensity values to a standard range (0–1 or 0–255).

  - Makes images consistent for feature extraction and classification.

  - Improves the performance of machine learning models.

Overall Preprocessing Pipeline

Raw Drone Image → Noise Reduction → Illumination Correction → Contrast Enhancement → Normalization → Feature Extraction

Justification

This preprocessing pipeline removes unwanted noise, corrects lighting variations, improves image contrast, and standardizes intensity values. As a result, pest-affected crop regions become more distinguishable, leading to more accurate feature extraction and classification.

b) Feature Extraction (3 Marks)

To identify pest-damaged crop regions, texture features provide valuable information because infected leaves often exhibit irregular patterns, rough textures, and discoloration.

Selected Method: SIFT (Scale-Invariant Feature Transform)

SIFT detects stable key points and generates descriptors that remain reliable even when the image undergoes changes in scale, rotation, or illumination.

Working Principle

- Detects key points using Difference of Gaussian (Dog).

- Assigns an orientation to each key point.

- Generates a feature descriptor based on local gradients.

- Matches descriptors between images.

Why SIFT is Suitable

- Detects fine texture variations caused by pest damage.

- Works well under different lighting conditions.

- Handles images captured from varying drone heights.

- Provides scale and rotation invariance.

- Produces highly distinctive feature descriptors.

Alternative Methods

- SURF: Faster than SIFT but slightly less accurate.

- ORB: Very fast and suitable for real-time systems but less robust for complex texture analysis.

Conclusion

SIFT is the most suitable feature extraction method because it accurately captures texture changes and remains reliable under varying imaging conditions.

c) Rotation and Scale Variations (3 Marks)

Drone images are captured from different heights, viewing angles, and camera orientations. These variations can significantly affect feature detection.

Effect of Rotation

If the drone rotates while capturing images:

- Features appear at different orientations.

- Traditional feature detectors may fail to match corresponding regions.

- Detection accuracy decreases.

Effect of Scale Variation

When the drone flies higher or lower:

- Crop regions appear larger or smaller.

- Features extracted at one scale may not match those extracted at another scale.

- Small pest-affected regions may disappear at lower resolutions.

Methods to Improve Robustness

1. Scale-Invariant Feature Transform (SIFT)

- Detects features across multiple image scales.

- Maintains accuracy despite zoom changes.

2. SURF

- Faster alternative to SIFT.

- Also provides scale and rotation invariance.

3. ORB

- Offers rotation invariance with lower computational cost.

- Suitable for real-time drone applications.

4. Image Pyramid

- Creates multiple scaled versions of the image.

- Detects features at different resolutions.

5. Data Augmentation

- Rotating and scaling training images helps machine learning models become more robust.

Conclusion

Using scale- and rotation-invariant feature extraction methods such as SIFT or SURF ensures reliable detection of pest-affected crop regions even when drone images are captured from different heights and angles.

Question 2: Stereo Vision and Motion Estimation

a) Stereo Disparity and Depth Estimation (4 Marks)

Stereo vision is a technique that uses two cameras placed at a fixed distance apart, similar to human eyes, to estimate the depth of objects.

Working Principle

- The left and right cameras capture images simultaneously.

- Corresponding points in both images are identified.

- The horizontal shift between matching points is called disparity.

- The depth of an object is calculated using the disparity value.

The depth equation is:

DepthZ=f×Bd

Where:

- f = Camera focal length

- B = Distance between the two cameras (baseline)

- d = Disparity

Disparity and Distance Relationship

- Nearby Objects

  - Large disparity.

  - Appear at significantly different positions in the two images.

  - Estimated depth is small.

- Distant Objects

  - Small disparity.

  - Appear almost at the same position.

  - Estimated depth is large.

Example

A nearby pedestrian produces a large disparity, allowing the system to recognize that the object is close and requires immediate attention. A distant vehicle has a smaller disparity, indicating it is farther away.

Importance in Self-Driving Cars

- Detects obstacles.

- Measures distances accurately.

- Supports collision avoidance.

- Assists in lane keeping and path planning.

b) Optical Flow for Motion Tracking (3 Marks)

Optical flow estimates the movement of pixels between consecutive video frames.

Working Principle

- Detects feature points on moving objects.

- Tracks their positions across successive frames.

- Computes motion vectors representing movement direction and speed.

Sudden Change in Speed

If a pedestrian starts running:

- Motion vectors become longer.

- Speed estimation increases.

- The system detects acceleration.

Sudden Change in Direction

If the pedestrian suddenly turns:

- Motion vectors change orientation.

- The tracking algorithm updates the predicted path.

- The vehicle adjusts its trajectory accordingly.

Advantages

- Enables continuous object tracking.

- Estimates object velocity.

- Predicts future positions.

- Helps avoid collisions.

Conclusion

Optical flow provides real-time information about object movement, enabling self-driving vehicles to safely respond to dynamic road conditions.

c) Errors and Mitigation Techniques (3 Marks)

Several factors can reduce the accuracy of stereo depth estimation and optical flow.

1. Camera Misalignment

Problem

- Cameras are not perfectly aligned.

- Incorrect disparity measurements occur.

Solution

- Perform stereo camera calibration.

- Use image rectification before disparity calculation.

2. Poor Lighting

Problem

- Shadows and low illumination reduce feature visibility.

- Matching between left and right images becomes difficult.

Solution

- Apply illumination correction.

- Use adaptive histogram equalization (CLAHE).

- Employ HDR imaging when available.

3. Occlusion

Problem

- One object blocks another.

- Corresponding points disappear.

- Optical flow loses tracking.

Solution

- Use Kalman Filters or Particle Filters.

- Employ multi-object tracking algorithms such as Deep SORT.

4. Fast Motion

Problem

- Large movement between consecutive frames.

- Optical flow may fail to establish correct correspondences.

Solution

- Use Pyramidal Lucas-Kanade Optical Flow.

- Increase camera frame rate.

5. Texture less Regions

Problem

- Plain walls or roads have few detectable features.

- Stereo matching becomes unreliable.

Solution

- Use feature detectors such as SIFT or ORB.

- Apply dense stereo matching algorithms.

- Combine stereo vision with LiDAR or depth sensors.

- Conclusion

Accurate depth estimation and motion tracking depend on high-quality images, proper camera calibration, reliable feature extraction, and robust tracking algorithms. By reducing errors caused by lighting, occlusion, fast motion, and texture less regions, self-driving vehicles can estimate object positions more accurately and make safer navigation decisions.